Clinical Feedback meeting notes by Goutam Reddy (greddy@abiogenix.com)

3/30/20 meeting @ Stanford Health (they run the drive-through swabbings)

Interviewed: Dr. Linda Barman, Thanh Khong PAC, Another Physician

Summary of Design Notes:

- felt no difference between Frozen vs. Room Temperature swabs

- Liked spiral design with recommended modifications from other designs:

  - spiral head w/ rounded tip (Barman design)

  - move neck-body interface down (to position of spring designs)

  - thicken breakpoint (okay to bend and twist to break off into tube)

  - move breakpoint down to 100mm

  - head of tip is fine, could be a little rounder

  - tip flimsiness: make sure doesn’t break when bent backwards (pushed against obstruction)

Swab Note:

They liked the 100mm break-off.  Once again WARNED that the 80mm break-off would be going into the nose of the patient, or would not reach nasopharynx.   <see video>

General Notes:

- They liked the tube testing:  said to test with more kinks/bumps to ensure simulation of blockages and constrictions.

- They tested by pushing the swab into the palm and backside of their hands to test flexibility

- They made a constriction of a hole in their hand and tried pushing into the constriction

- They clenched a fist and twisted in their hand

- They also twisted and pulled and bent swab to test material strength

- All colleagues suggested a slight curve to the neck

Test 21)  HP Spring 80mm (mfg by HP in San Diego)

- autoclaved at 121 vs 134: no difference

- frozen vs room temperature:  no difference

- Like head; end should be rounder (tip rounder)

- snapped neck and snapped head (too brittle, “more brittle”)

- make it stiff, but not brittle

- “will jam up” inside nose

- “too soft, jam up and not go”

- Spring is too floppy, “too floppy”

- Stiffen shaft a little

- Spring harder to get through

Test 22) HP Spring 100mm (mfg by HP in San Diego)

- autoclaved at 121 vs 134: no difference

- frozen vs room temperature:  no difference

- spring way too wiggly; like spiral because it is stiffer

- spring too bendy

- neck too skinny -> make thicker

- want a little stiffer than spring, but not breakable like spiral

- like breakpoint placement

Test 23) Fathom Squares in PA12 (mfg by HP in San Diego)

- not rounded enough

- stiffer than spring

- like thickness of neck

- make the breakpoints (for all) harder to break;

  - if you have to bend and twist, its no problem

- the cutdown is too thin

Test 24) Barman-Fathom Spiral in PA12 (mfg by HP in San Diego)

- like this head the best (not as stiff as squares, not as flimsy as springs)

- tip okay, but round out tip even more if you can

- neck not long enough = make longer

- broke neck by flexing

- breakpoint is too deep of a cut => make stronger

  - don’t’ want to snap off inside someone’s nose

- move breakpoint to 100mm design